Is The Transition To Zero Carbon Power Economically Feasible? The Case Of A 70% Variable Renewables Power System

Sarah Stanley, School of Electrical and Electronic Engineering, University College Dublin, 00353-1-716-1884, sarah.stanley1@ucdconnect.ie

Rinalini Lahon, School of Electrical and Electronic Engineering, University College Dublin, 00353-1-716-1884, rinalini.lahon@ucd.ie

Ciara O’Dwyer, School of Electrical and Electronic Engineering, University College Dublin, 00353-1-716-1884, ciara.odwyer@ucd.ie

Lisa Ryan, School of Economics, University College Dublin, 00353-1-716-8212, lisa.ryan@ucd.ie

Damian Flynn, School of Electrical and Electronic Engineering, University College Dublin, 00353-1-716-1819, damian.flynn@ucd.ie

## Overview

## Ambitious national renewable electricity targets in Europe have resulted in countries without significant hydro resources, such as Denmark, Ireland and Spain, aiming to source over 70% of their annual electricity consumption from variable renewables by as early as 2030. A high share of wind and solar power in the electricity system introduces numerous technical and operational issues, as well as raising questions about the financial viability of generators in the current electricity market. The objective of this paper is to examine whether such high shares of renewable electricity are economically feasible, under the current electricity market design and technical constraints, and the policy implications.

## Methods

## We apply a scenario-based, system-scheduling approach, using the unit commitment model Backbone. System security and renewables curtailment under various renewable generation and electricity load profiles and scenarios are considered, as are market prices and net present values of different technology types in 2030.

## Results

## We find that high shares of variable renewable electricity in isolated systems such as Ireland could lead to curtailment of up to 15% but that additional interconnection, battery storage and dynamic load behaviour reduce this. Marginal prices fall to near zero in periods of high generation of renewable electricity. We estimate that this, combined with curtailment, may lead to insufficient revenues from the electricity price alone in existing electricity markets to cover the costs of generation for many renewable and conventional power plants.

## Conclusions

## The design of electricity markets needs to ensure additional revenue streams from capacity markets and tariffs designed for system services incentivise investment in system security and make future decarbonised electricity systems economically feasible.

## References

IEA (2019), World Energy Outlook 2019, OECD Publishing, Paris, https://doi.org/10.1787/caf32f3b-en.

[2] L. Söder, E. Tómasson, A. Estanqueiro, et al.: “Review of wind generation in adequacy calculations and capacity markets in different systems”, Renewable and Sustainable Energy Reviews, vol. 11, March 2020.

[3] Holttinen, H., Kiviluoma, J., Levy, T., et al. “Design and operation of power systems with large amounts of wind power.” Final summary report, IEA WIND Task 25, Phase four 2015–2017, 2019.

[4] Hodge, B-M. et al., “Addressing technical challenges in 100% variable inverter-based renewable energy power systems,” WIREs Energy and Environment, May 2020.

[5] ENTSOE European Power System 2040, Completing the map, System Needs Analysis, part of ENTSO-E’s 2025, 2030, 3040 Network Development Plan 2018. [Online]. Available:

https://eepublicdownloads.blob.core.windows.net/public-cdn-container/clean-documents/tyndp-documents/TYNDP2018/european_power_system_2040.pdf

[6] Mehigan, L., Al Kez, D., Collins, S., Foley, A., Ó’Gallachóir, B., Deane, P. "Renewables in the European power system and the impact on system rotational inertia". Energy, vol. 203, July 2020.

[7] Johnson, S., Rhodes, J., Webber, M, “Understanding the impact of non-synchronous wind and solar generation on grid stability and identifying mitigation pathways,” Applied Energy, vol. 262, March 2020.

[8] IRENA, Electricity Storage Valuation Framework: Assessing system value and ensuring project viability. International Renewable Energy Agency, Abu Dhabi. 2020. [Online]. Available: https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2020/Mar/IRENA_storage_valuation_2020.pdf

[9] EirGrid Group, “The DS3 Programme: Delivering a Secure, Sustainable Electricity System,” Tech. Rep. [Online]. Available: http://www.eirgridgroup.com/sitefiles/library/EirGrid/DS3-Programme-Brochure.pdf

[10] Blazquez, J., Fuentes-Bracamontes, R., Bollino, C.A., Nezamuddin, N., “The renewable energy policy Paradox”, Renewable and Sustainable Energy Reviews, vol. 82, Part 1, pp 1-5, 2018.

[11] Matsuo, Y., Endo, S., Nagatomi, Y., et al., “Investigating the economics of the power sector under high penetration of variable renewable energies,” Applied Energy, vol. 267, 2020.

[12] Gerres, T., Chaves Avila, J.P., Martines, F.M., et al. “Rethinking the electricity market design: Remuneration mechanisms to reach high RES shares. Results from a Spanish case study”, Energy Policy, vol. 129, pp. 1320-1330, 2019.

[13] Gaffney, F., Deane, P., Ó. Gallachóir, B., “Reconciling high renewable electricity ambitions with market economics and system operation: Lessons from Ireland's power system,” Energy Strategy Reviews, vol. 26, 2019.

[14] J. Sullivan, A. Rogers, D. Flynn, et al., “Studying the maximum instantaneous non-synchronous generation in an island system-frequency stability challenges in Ireland,” IEEE Trans. Power Syst., vol. 29, no. 6, pp. 2943-2951, Nov. 2014.

[15] Government of Ireland, “Climate action plan 2019,” Government of Ireland, 2019.

[16] N. Helisto, J. Kiviluoma, J. Ikaheimo, et al., “Backbone: An adaptable energy systems modelling framework,” Energies, vol. 12, no. 17, pp. 1-35, 2019.

[17] EirGrid Group, “Operational Constraints Update 29/03/2019,” Tech. Rep., 2019. [Online]. Available: http://www.eirgridgroup.com/sitefiles/library/EirGrid/Operational-Constraints-Update-Mar 2019.pdf

[18] Department for Business, Energy & Industrial Strategy, “BEIS 2019 Fossil Fuel Price Assumptions,” BEIS, 2019.

[19] M.T. Devine and M. Russo, “Liquefied natural gas and gas storage valuation: Lessons from the integrated Irish and UK markets,” Applied Energy, vol. 238, pp. 1389-1406, 2019.

[20] EirGrid, “Tomorrow’s Energy Scenarios 2019 Ireland,” EirGrid, 2019

[21] SONI, “Tomorrow’s Energy Scenarios Northern Ireland 2019 Consultation,” SONI, 2019.

[22] I. Staffell and S. Pfenninger, “Using bias-corrected reanalysis to simulate current and future wind power output,” Energy, vol. 114, pp. 1224-1239, 2016.

[23] Ruhnau, O., Hirth, L., and Praktiknjo, A. “Time series of heat demand and heat pump efficiency for energy system modelling,” Scientific Data, 6, 189. 2019.

[24] Hedegaard, K., Mathiesen, B. V., Lund, H., & Heiselberg, P. “Wind power integration using individual heat pumps – Analysis of different heat storage options”. Energy, 47, 284-293. 2012. https://doi.org/10.1016/j.energy.2012.09.030

[25] Element Energy, “Electric Vehicle Charging Behaviour Study,” Report commissioned by the National Grid ESO, Element Energy, Cambridge, March 2019

[26] Newbery, D. “Implications of the National Energy and Climate Plans for the Single Electricity Market of the island of Ireland,” Cambridge Working Paper in Economics 2072. 2020.

[27] López Prol, J., Steininger, K.W., Zilberman, D., “The cannibalization effect of wind and solar in the California wholesale electricity market”. Energy Economics, 85, 10455. 2020.

[28] Ó Gallachóir B.P., Chiodi A., Gargiulo M., Deane J.P., Lavigne D. and Rout U.K. Irish TIMES Energy Systems Model. Report published by the EPA. 2013.

[29] Grant Thornton, “Renewable energy discount rate survey results – 2018”, January 2019. [Online]. Available: https://www.grantthornton.ie/insights/publications/renewable-energy-discount-rate-survey-results--2018/

[30] C. O’Dwyer, L. Ryan, and D. Flynn, “Efficient large-scale energy storage dispatch: challenges in future high renewable systems,” IEEE Trans. Power Syst., 32, 5, 3439–3450. 2017