# VALUATION OF MANAGERIAL FLEXIBILITY AND STRATEGIC VALUE OF COAL GASIFICATION PROJECTS WITH THE OPTION-TO-SWITCH

### Piotr W. Saługa, AGH University of Science and Technology of Cracow, Phone +48 12 617 43 22,

### E-mail: psaluga@zarz.agh.edu.pl
Paweł Grzesiak, AGH University of Science and Technology of Cracow, Phone +48 12 617 42 82,

### E-mail:pgrzesia@agh.edu.pl

### Jacek Kamiński, MEERI PAS of Cracow, Phone +48 12 633 02 96, E-mail: kaminski@meeri.pl

#### Overview

#### In recent years, coal gasification has been popularized as an advanced clean energy alternative to coal combustion. Furthermore, the interests of the power generation sector, which is constantly seeking for attractive environmentally friendly solutions, has promoted the development of innovative coal gasification technologies. Aside from the usual technical problems (e.g. design, reaction kinetics) which require complex problem-solving skills, engineers have to assess the efficiency and economic viability of these risky technologies. This task is not trivial due to the substantial amounts of managerial flexibility that need to be valued and understood. The flexibility arises from the specific features of the coal gasification technologies including, e.g., management's ability to defer the start-up of the project or – at the operational stage – expand/shrink, temporarily shut-down/reopen production, or switch between alternative products. The switching possibilities seem to be particularly interesting.

On repeated occasions, it has been found that classical discounted cash flow techniques could lead to incongruent adjustments to account for risk and, in consequence, undervalue projects with high extent of flexibility. Its value can be captured by modern methods such as real options analysis. In addition to allocating value to flexibility, real options valuation methods deliver decision-making criteria and formulate policies to maximize a project’s value.

The purpose of this paper is to present the specifics of coal gasification projects and highlight the importance of ‘switching’ possibilities within the gasification technology.

The paper is organized as follows: the introduction paragraph provides an overview of coal gasification technologies and real options analysis. The second section delivers the developed valuation methodology for gasification projects. The third section presents the results and discussion. Finally, the paper closes with summary and conclusions.

#### Methods

Discounted cash flow analysis. Logarithmic cash flow returns method. Real options analysis – a risk-adjusted binomial tree.

#### Results

Firstly, combination of two type switching options makes a significant contribution to value creation.

Secondly, real options analysis delivers high strategic value to coal gasification projects, when compared to the traditional discounted cash flow analysis.

Thirdly, applying real options analysis to a gasification project changes the conventional decision-making criteria.

#### Conclusions

#### Managerial flexibility, inherent to coal gasification technologies, has a specific financial dimension. Real options analysis delivers a robust and credible value to a coal gasification project, when compared to the traditional discounted cash flow analysis, since it allocates value to flexibility. The presented valuation algorithm of two variants of the switching option (i.e. option to mothball/restart production and option-to-switch between different final products) is practical and captures – in the context of economic evaluation – uncertain characteristics of the coal gasification technology. Hence, the real options approach can be applicable to other similar valuation problems.

#### References

Abadie L.M. and J.M. Chamorro (2008): „Valuation of Energy Investments as Real Options: The case of an Integrated Gasification Combined Cycle Power Plant”, Energy Economics 30.

Abadie L.M. and J.M. Chamorro (2009): „The Economics of Gasification: A Market-Based Approach”, Energies 2.

ARE (2014): „Sytuacja Techniczno-Ekonomiczna Sektora Elektroenergetycznego”, Agencja Rynku Energii S.A. (Polish language)

Białecka B. (2008): „Podziemne zgazowanie węgla. Podstawy procesu decyzyjnego”, Publishing House: GIG, Katowice. (Polish language)

Burton E., Friedmann J. and R. Upadhye (2004): „Best Practices in Underground Coal Gasification”, U.S. Department of Energy by the University of California, Lawrence Livermore National Laboratory Contract No. W-7405-Eng-48.

Chmielniak T. and M. Ściążko (2007): „Technologie zgazowania węgla”, KARBO. (Polish language)

Chmielniak T. and M. Sciążko (2008): „Zgazowanie węgla” [in:] Borowiecki T., Kijeński J., Machnikowski J. and M. Ściążko (Editors): Czysta energia, produkty chemiczne i paliwa z węgla – ocena potencjału rozwojowego, Puublishing House: Wydawnictwo Instytutu Chemicznej Przeróbki Węgla, Zabrze. (Polish language)

Copeland, T. and V. Antikarov (2001): „Real Options: A Practitioner’s Guide”, Texere, Thompson Corporation, pp. 370.

Copeland, T., Koller, T. and J. Murrin, (2000): „Valuation: Measuring and Managing the Value of Companies” (3rd Edition), Publishing House: John Wiley and Sons, New York.

Creedy D.P. and K. Garner (2004): „Clean Energy from Underground Coal Gasification in China”, DTI Cleaner Coal Technology Transfer Programme, Report No. COAL r2R250, DTI/Pub URN 03/1611.

Dubiński J. and K. Stańczyk (2010): Podziemne zgazowanie węgla – doświadczenia światowe i eksperymenty prowadzone w KD Barbara. Szkoła Eksploatacji Podziemnej, Materiały Konferencyjne, Kraków. (Polish language)

Główny Urząd Statystyczny: „Gospodarka paliwowo-energetyczna w latach 2013 i 2014”, Warszawa, GUS, http://stat.gov.pl/ (Polish language)

Greg D.W. and T.F. Edgar (1978): „Underground Coal Gasification”, AIChE Journal, Volume 24, Issue 5, p. 753-781.

Hajdo S., Kasztelewicz Z. and K. Polak (2010): „Perspektywy, ograniczenia oraz zadania dla wdrożenia podziemnego zgazowania węgla w Polsce w świetle doświadczeń światowych”, SITG Conference, Ustroń. (Polish language)

He Y. (2007): „Real Options in the Energy Markets”, PhD Dissertation, University of Twente, Netherlands.

Herbelott O. (1994): „Option Valuation of Flexible Investments: the Case of a CoalGgasifier”, Massachusetts Institute of Technology, January.

Hyncar J. (2008): „Kierunki rozwoju procesów zgazowania paliw”, KARBO. (Polish language)

IChPW (2008): „Czysta energia, produkty chemiczne i paliwa z węgla – ocena potencjału rozwojowego”, Praca zbiorowa, Wydawnictwo IChPW, Zabrze. (Polish language)

Kwaśniewski K., Kopacz M., Grzesiak P. and R. Kapłan (2014): „Ekonomiczna ocena technologii zgazowania węgla ze szczególnym uwzględnieniem sekwencji składanej opcji czekania i wzrostu skali”, The Polish Mining Review 70/11, p. 97-106. (Polish language)

Kwaśniewski K., Kopacz M., Grzesiak P., Kapłan R. and D. Kryzia (2016): „Wytwarzanie metanolu na drodze zgazowania węgla – perspektywa ekonomiczna”, Przem. Chem. 95/1, p. 1000-07. (Polish language)

Laurikka H. (2006): „Option Value of Gasification Technology within an Emissions Trading Scheme”, Energy Policy, 34, Issue 18, December.

Nexant, Inc. (2006): „Environmental Footprints and Costs of Coal-Based Integrated Gasification Combined Cycle and Pulverized Coal Technologies”, EPA-430/R-06/006.

Reedman L., Graham P. and P. Coombes (2006): „Using a Real Options Approach to Model Technology Adoption under Carbon Price Uncertainty: An Application to the Australian Electricity Generation Sector”, Economic Record, Vol. 82, No. S1.

Reedman L., Graham P., Coombes P. and D. Vincent (2006): Impact of Carbon Price Uncertainty on Investment in Selected Electricity Generation Options, Technology Assessment Report, 59.

Saługa P. (2011): „Elastyczność decyzyjna w procesach wyceny projektów geologiczno-górniczych”, Studia, Rozprawy, Monografie, Publishing House: IGSMiE PAN, Kraków. (Polish language)

Sekar Ch.R. (2005): „Carbon Dioxide Capture from Coal-Fired Power Plants: A Real Options Analysis”, MSc Dissertation, Massachusetts Institute of Technology.

Smeers Y., Bolle L. and O. Squilbin (2001): „Coal Options: Evaluation of Coal-Based Power Generation in an Uncertain Context” – Final report, OSTC ¬ Global Change and Sustainable Development 1996-2000, Sub-programme 2.

Szuba J. and L. Michalik (1983): „Karbochemia – zarys rozwoju”, Publishing House: „Śląsk”. (Polish language)

Teng Y., Han L., Li Ch. and H. Zhao (2011): „Real Option Analysis on Coal-to-Oil Project”, Energy Procedia 5.

Trigeorgis, L. (1996): „Real Options – Managerial Flexibility and Strategy in Resource Allocation”, The MIT Press, Cambridge, Massachusetts – London, England, pp. 427.

UCG Engineering, Ltd. (2006): „Underground Coal Gasification: Basic Concepts”, http://www.coal-ucg.com/concept2.html

U.S. Department of Energy, NETL (2010): „Cost and Performance Baseline for Fossil Energy Plants”, Volume 1: Bituminous Coal and Natural Gas to Electricity, DOE/2010/1397, www.netl.doe.gov.

Yang M. and W. Blyth (2007): „Modelling Investment Risks and Uncertainties with Real Options Approach”, Paper Number LTO/2007/WP01, Paris.

Yun T. and R. Baker (2009): „Analysis of a Power Plant Investment Opportunity Under A Carbon Neutral World”, Investment Management and Financial Innovations, Volume 6, Issue 4, www.methanex.com