- Johannes Herold1, Pao-Yu Oei2, Andreas Tissen3

- Modeling a Carbon Capture, transport and Storage Infrastrstucture for the Industrial Sector

- 1Workgroup for Economic and Infrastructure Policy, Technische Universität Berlin, Germany, +49 30314 23252, jh@wip.tu-berlin.de

- 2Workgroup for Economic and Infrastructure Policy, Technische Universität Berlin, Germany, +49 30314 27500, paoyuoei@gmail.com

- 3German Institute for Economic Research Berlin, Germany, +49 30 89789 222, atissen@diw.de

# Overview

Various macroscopic analyses such as the IEA CCS Roadmap have indicated that Carbon capture, transport and storage (CCTS) is an important part of the lowest-cost greenhouse gas (GHG) mitigation portfolio [3].

In 2010, the German government announced an 80% CO2 reduction until the year 2050. This ambitious target requires means far behind efficiency improvements or a switch in fuels, both for the energy and industrial sector.

Industrial sectors, such as iron and steel production, cement manufacturing; refineries and the chemical industry are characterized by a high CO2 intensity in production. Those emissions are partly based on the combustion of fossil fuels but also on the underlying processes. Yet in contrast to the energy sector, near- or mid-term substitutes to CO2 intensive processes are not in the offing and modern facilities often work close to the maximum of the physical efficiency.

By applying the CCTS technologies, 80 – 90% of the CO2 emission could be captured. For the steel and cement industry, capturing could even be accomplished at lower costs compared to power plants. But due to the structure of the sector, which is characterized by a high number of small and regionally dispersed emission sources, complexity of a future CCTS infrastructure would increase, mainly to a longer pipeline network.

Given the limited storage space raises the question, whether the energy sector should also be part of a future CCTS infrastructure at all. This could raise network efficiency du to economies of scale in the pipeline transport, but would sharpen the scarcity of storage potential.

This paper presents a geo-economic model aiming at a microscopic evaluation of the development of a CCTS infrastructure from a German and European perspective. We test the influence of different assumption on the model results e.g., the storage potential, the CO2 price, and a solely adoption of the technology by the industrial sector.

# Methods

The analysis is based on the CCTSMOD-model, which was presented at the IAEE conference at Vilnius in 2010. CCTSMOD is a mixed integer approach of a multi period, welfare optimizing CCTS network model. It can be used to support decisions on implementation of a CCTS infrastructure. The model includes endogenous decisions about pipeline investments, carbon capturing investments, CO2 ejection and flow quantities based on given costs, a CO2 certificate price, capacities and emissions for every power plant in the data set. Sources and sinks of CO2 are aggregated to nodes according to their geographic position. Pipelines can only be constructed between neighboring or diagonal nodes. The distance between two neighboring node can be chosen freely. Thus the model is scalable with regards to the resolution.

The novelty of this approach is that it explicitly takes into account CCTS in the industrial sector. We have the option to combine the analysis of CCTS in the energy sector and the industrial sector, but the model can also be confined to the industrial sector only. Various scenarios can be developed that sketch out a precise methodological basis for decision-making in the industrial sector, that has so far been largely ignored by scientific analysis and policy.

# Preliminary Results

Previous results have shown that the industrial sector acts as a first mover for the CCTS adoption [5]. In a welfare optimizing setting, trunk line pipelines are built in early periods which are also used by the energy sector in later periods. A cost-optimal CCTS infrastructure can be described as a number of regional networks evolving around distinct storage facilities with no connection between the networks. Results also reveal that even under high CO2 prices only parts of the industrial and energy sector would use the technology. A pipeline network of 4400 km in 2050 leads to an average annual storage of 200 Mt, or 34% of the total emitted CO2 from large point sources in Germany.

An emission reduction of 90% is achievable if the German industrial sector is forced into applying CCTS. For Germany, this requires construction of 10.222 km pipelines (worth 2 bn €) and investment into capture of more than 6 bn € to store an annual amount of 50 mil t/CO2. This highlights the increased complexity of a solely industrial application of the technology where the focus is not longer set on capture, but on the transport infrastructure.

# References

- Dooley J. J. et al. (2006). Carbon Dioxide Capture and Geological Storage - a core element of a global energy technology strategy to address climate change, Global Energy Technology Strategy Program (GTSP). - College Park, USA: Battelle, Joint Global Change Research Institute.

- Fritze K. (2009). Modeling CO2 Storage Pipeline Routes in the United States, Master's Project ; Nicholas School of the Environment . - USA : Duke University.

- International Energy Agency (2009). Technology Roadmap - Carbon Capture and Storage. - Paris, France : International Energy Agency..

- Middleton R. and Bielicki J. A (2009). Comprehensive Carbon Capture and Storage Infrastructure Model , Proceedings of the 9th International Conference on Greenhouse Gas Technologies.

- Roman Mendelevitch, Johannes Herold, Pao-Yu Oei and Andreas Tissen: CO2 Highways for Europe Modelling a Carbon Capture, Transport and Storage Infrastructure for Europe. CEPS Working Document No. 340/November 2010