HYDROGEN FUEL CELL POWERTRAIN LEVELIZED COST OF ELECTRICITY

Mario Valentino Romeri, Independent Consultant, +39 339 4980033, valentino.romeri@alice.it

## Overview

## The notion of Levelized Costs of Generating Electricity (LCOE) is a handy tool for comparing the unit costs of different power generation technologies.

## The Vehicle-to-Grid concept is well known but, in this paper, the vehicle power generation system hydrogen fuel cell based (H2FC powertrain) was considered not only in V2G perspective, but as a power generation plant, smart grid connected.

## Based only on public data, H2FC powertrain LCOE was compared with the traditional power generation technologies LCOE in the U.S. context.

## Methods

## In LCOE financial model, different cost components are taken into account: capital costs, fuel costs, operations and maintenance costs (O&M). These costs are an average over the life of a project and for a specific technology, based on a specific and particular set of assumptions. The costs cash-ﬂow is discounted to the present (date of commissioning) using assumed specific discount rates. The resultant LCOE values, one for each generation option, are the main driver for choice technology.

## In mass production perspective, H2FC Powertrain will be so cost competitive to be useful adopted also for stationary power generation application. Using only DOE public data (Current or Targets) I calculated the H2FC Powertrain specific LCOE cost range, then I compared the H2FC Powertrain specific LCOE cost range with the generation costs of the traditional power generation technologies in the . U.S. context (LCOE data published by EIA).

## Results

## Results were surprising. Using 2009-2010 US DOE H2FC powertrain data (referred to high projected production volume) I found that the LCOE would be in a range of USD 175 - 192 for MWh. Using the 2015 US DOE data target the H2FC powertrain cost range moves to USD 107-207 for MWh, and, for the lower value of this range, it appears competitive with many of the power generation technologies considered.

## Conclusions

If the current US Hydrogen and Fuel Cell Vehicle Program is able to meet all the 2015 technological targets the high volume associated with the H2FC vehicles mass production will permit to reduce dramatically the Fuel Cell system manufacturing costs and the H2FC Powertrain will be so cost competitive to be useful adopted also for stationary power generation application.

Observing the H2FC Powertrain data, it will be necessary to think the Hydrogen FCV link to energy sector considering also the possibility to utilize the H2FC Powertrain as a Power Generation Plant, smart grid connected, with relevant and positive consequences for a rapid development of these low-carbon technologies.

## References

The concept of ‘Vehicle-to-Grid’ was first proposed in 1997 by W. Kempton in his article “Electric vehicles as a new power source for electric utilities”, Transportation Research Part D 2 (3), 1997, p157–175 < http://www.udel.edu/V2G/docs/Kempton-Letendre-97.pdf >.

W. Kempton, J. Tomić "Vehicle to Grid Implementation: from stabilizing the grid to supporting large-scale renewable energy", Journal of Power Sources Volume 144, 2005 < http://www.spinnovation.com/sn/Articles_on_V2G/Vehicle-to-grid_power_implementation_From_stabilizing_the.pdf >.

W. Kempton, J. Tomić, “Vehicle-to-grid power fundamentals: Calculating capacity and net revenue”, Journal of Power Sources, Volume 144, 2005 < http://www.spinnovation.com/sn/Articles_on_V2G/Vehicle-to-grid_power_fundamentals_Calculating_capacity.pdf >.

S. Satyapal “Hydrogen and Fuel Cell Technologies Update” Fuel Cells & Hydrogen Joint Undertaking Stakeholders General Assembly November 2010, Brussels < http://ec.europa.eu/research/fch/pdf/sunita_satyapal.pdf#view=fit&pagemode=none >.

K. Wipke “Controlled Hydrogen Fleet and Infrastructure Analysis”, US DOE NREL, Annual Merit Review, Washington, 2010, < http://www.hydrogen.energy.gov/pdfs/review10/tv001_wipke_2010_o_web.pdf >.

US Energy Information Administration (EIA) “2016 Levelized Cost of New Generation Resources from the Annual Energy Outlook 2010” Washington, USA January 2010, < http://www.eia.gov/oiaf/archive/aeo10/electricity_generation.html >.

US EIA “Levelized Cost of New Generation Resources in the Annual Energy Outlook 2011” < http://www.eia.doe.gov/oiaf/aeo/pdf/2016levelized_costs_aeo2011.pdf  >.