analysis of The impacts of co2 capture and shale gas supply under a co2 tax scenario

Christopher Nichols, Phone +1 304 285 4172, E-mail: Christopher.Nichols@ netl.doe.gov

Nadejda Victor, Phone +1 412 386 5152, E-mail:  Nadejda.Victor@contr.netl.doe.gov

Peter Balash, Phone +1 412 386 5753 , E-mail: Peter.Balash @netl.doe.gov

[Format: single space, 10 point font, Times New Roman]

## Overview

Security of energy supply and climate change are central concerns for policy makers and important dimensions of long-term energy sustainability. This paper examines the role of policy instruments in managing climate change and stimulating technological change towards a sustainable energy system in the long-term future. The approach, which makes use of the MARKet ALlocation (MARKAL) energy system model, allows policy instruments to be examined quantitatively in a dynamic energy system context.

The study is focused on the interaction between climate policies and energy security, and will help guide policy making by identifying areas, and the extent to which, climate policy can reinforce energy security objectives.  The study discusses multiple scenarios, considering the synergistic effects of energy development objectives, diversification in energy supply and environmental regulations in the U.S. at the regional level.

The paper is organised as follows: After the introduction the second section gives a brief overview on the MARKAL) energy system model, the data sources and  methods. The third section addresses the results.  In the final section policy implications are derived.

## Methods

The primary objective of the analysis is to evaluate the techno-economic impacts of the CO2 taxation on the U.S. energy system, with a particular emphasis on the electricity sector. The paper presents the range of findings from a selection of model runs undertaken at the regional levels.

We adopt MARKAL because it is the most widely-applied energy-economy model in literature, and the latest, 2012, US EPA 9 Regions MARKAL database (EPAUS9r 2012) is available. MARKAL is a linear program that has energy producing, conversion, and use activities and capacities as decision variables, and constraints representing energy balances, capacity limits, and various policy considerations. The objective function is to minimize the cost of meeting those demands and this formulation simulates the operation of a competitive market. The analyses address only a subset of the CO2 taxation policy and possible effects on CO2 emissions reduction, energy costs and energy security. Energy security has multiple features, which cannot be easily combined into a single indicator, so energy security is not used as a quantitative target or a technical modelling constraint in the MARKAL’s scenarios.

## The EPAUS9r2012 database was slightly modified for one scenario (we changed the CCS technologies profile and included learning rates for coal and gas plants with CCS). The learning rates were derived from the Energy Information Administration’s (EIA) publication, Electricity Market Module of the National Energy Modeling System 2012 (see DOE/EIA-M068, 2012).

We analysed electricity mix, abatement costs, marginal energy costs at the residential sector and energy security under different scenarios projections. Long-term energy supply security is contingent on the establishment of “efficient” diversified portfolios of primary and secondary energy sources. A more diverse system is perceived as having a number of benefits that make it preferable to one that is less diverse. In particular, diversity is considered to contribute to achieving energy security since disruption of any one source will have a smaller impact on overall energy supply. Similarly, the effects of price volatility are likely to be mitigated where an increasing range of sources is employed in energy production.

We have examined the techno-economic impacts of these two climate change policies on carbon emissions reductions, electricity generation mix and energy security through analyzing various assumptions using the following scenarios (see Table 1):

Table 1. Scenario Definitions

| Scenarios | Descriptions |
| --- | --- |
| BASE9R | Base case (BASE9r scenario from the EPAUS9r 2012 database) |
| BASE9RVT | Base case with CAIR-MATS |
| BASECO2 | Base case with CAIR-MATS and CO2 taxes from Sanders/Boxer Climate Legislation ($20/tCO2 in 2015 with 5.6% annual growth) |
| CO2CCS | BASECO2 scenario with learning assumptions for CCS technologies. |
| CLOWGAS | BASECO2 scenario with lower EUR for shale gas |
| CLGCCS | BASECO2 scenario with lower EUR for shale gas and improved CCS technologies |

## Results

Results are presented in scenario format at the national and regional levels. The modelling scenarios results are compared to each other and a base case scenario. The reduced diversity in electricity input fuels is an important challenge in some scenarios. One aspect that is important to examine is whether gas and electricity markets continue to provide the required level of fuel diversity. In addition, diversification requires that all energy options keep open. The assessment of future energy scenarios shows that it is technically possible to achieve energy security while avoiding dangerous climate change. Electricity production provides a snapshot of both the effectiveness of CO2 mitigation actions and the level of energy security.

## Conclusions

There are the following substantial uncertainties facing the U.S. energy system: the risk of high energy prices, the risk of a disruption to energy supply and the risk of anthropogenic climate change. Failure to manage either of these risks may potentially result in large economic and social costs, so policies that effectively manage and reduce these risks may result in very large social benefits. We have compared policies that enhance long-term GHG emissions reductions, examined how these policies may interact with energy security, and the role of particular technologies in pursuing these policy goals.

As global gas demand and trade are expected to grow significantly, gas security will become increasingly important, not only for the gas industry but also for the energy industry. The expected increase in gas demand does not in itself have to constitute a security problem, provided that adequate security measures are in place. However, the larger the share of gas in the fuel balance, the more important gas security becomes.

The reduced diversity in electricity input fuels is rapidly becoming another important challenge. One aspect that needs to be watched carefully is whether gas and electricity markets continue to provide the required level of fuel diversity after they become deregulated. Diversification also requires that all energy options be kept open, including the nuclear power that is now facing a moratorium in several countries. Last but not least, efforts should be made to maintain sufficient operating margin, increase interconnections and technical compatibility among various networks and reduce barriers to cross-border trade in gas and electricity.

Identification of the consequences of these design features and alternative approaches requires additional modeling and analysis. However, it is clear that fossil fuels plants with CCS will play an important part under any GHG regulations strategy that address considerable CO2 reduction and acceleration support and funding for the large‐scale CCS demonstrations is crucial for the policies implementation. CCS technologies mitigated the increase in electricity prices and reduction in overall electricity generation driven by the various CO2 reduction scenarios we examined. Delays in CCS funding decisions that are caused by a number of factors must be addressed.

## References

Angelopoulos, K., Economides, G., Philippopoulos, A. (2010), What is the Best Environmental Policy?, Taxes, Permits and Rules under Economic and Environmental Uncertainty, CESifo Working Paper Series No. 2980, March  2010, http://papers.ssrn.com/sol3/papers.cfm?abstract_id=1571058,

DB (2010), Deutsche Bank Group Climate Change Advisors, Natural Gas and Renewables: A Secure Low Carbon Future Energy Plan for the United States, Deutsche Bank, online: http://www.anga.us/media/180381/deutsche%20bank%20report-%20nov%202010.pdf

Duncan, A. (1999), Economic Instruments for Pollution Control and Prevention – A Brief Overview, World Resources Institute, September, 1999, http://pdf.wri.org/incentives_austin.pdf. Accessed 29 May 2012

Bovenberg, A. L. & Goulder, L.H. (2002), Environmental taxation and regulation, Handbook of Public Economics, in: A. J. Auerbach & M. Feldstein (ed.), Handbook of Public Economics, edition 1, volume 3, chapter 23, pages 1471-1545 Elsevier.

EIA (2010a), Updated Capital Cost Estimates for Electricity Generation Plants, online: http://www.eia.doe.gov/oiaf/beck_plantcosts/pdf/updatedplantcosts.pdf

EIA (2010b), Electricity Market Module, April 2010, online: http://www.eia.gov/oiaf/aeo/assumption/pdf/electricity.pdf

EIA (2011a),  Assumptions to the Annual Energy Outlook 2011, U.S. Energy Information Administration, July 2011, online: ftp://ftp.eia.doe.gov/forecasting/0554(2011).pdf

EPA (2005),  Standalone documentation for EPA base case 2004 (V.2.1.9) using the Integrated Planning Model, Washington DC (September 2005), http://www.epa.gov/airmarkt/progsregs/epa-ipm/docs/bc1intro.pdf

EPA (2006), U.S. Environmental Protection Agency (2006). EPA U.S. National MARKAL Database: Database Documentation. Shay, C.L., J.F. DeCarolis, D.H. Loughlin, C.L. Gage, S. Yeh, S. Vijay, E.L. Wright. Feb 2006.

EPA (2011), Regulatory Impact Analysis for the Federal Implementation Plans to Reduce Interstate Transport of Fine Particulate Matter and Ozone in 27 States, U.S. EPA Office of Air and Radiation, June 2011, online: http://www.epa.gov/airtransport/pdfs/FinalRIA.pdf

EPA (2012), Regulations and Statutes, http://www.epa.gov/lawsregs/sectors/, Accessed 29 May 2012Gillingham, K. (2008), Kenneth Gillingham , Richard G. Newell, and William A. Pizer, Modeling endogenous technological change for climate policy analysis Energy Economics Volume 30, Issue 6, November 2008, Pages 2734-2753

Goulder, L. H. & Parry, I. W. H., Roberton, W,  Burtraw, D. (1999), The cost-effectiveness of alternative instruments for environmental protection in a second-best setting,  Journal of Public Economics, Elsevier, vol. 72(3), pages 329-360, June

H.R.5575 (2008), Moratorium on Uncontrolled Power Plants Act of 2008, online: http://www.opencongress.org/bill/110-h5575/show

Hugh Wynne et al.(2010), U.S. Utilities: Coal-Fired Generation Is Squeezed in the Vice of EPA Regulation; Who Wins and Who Loses?, Bernstein Research, online: http://207.114.134.6/coal/oh/downloads/bernstein-report.pdf

IEA (2012), Golden Rules for a Golden Age of Gas, World Energy Outlook Special Report on Unconventional Gas, International Energy Agency, OECD/IEA, 2012

I

CF (2010c), Coal-Fired Electric Generation Unit Retirement Analysis, ICF prepared for Interstate Natural Gas Association of America, online: http://www.ingaa.org/File.aspx?id=10371

ICF (2010a), Preliminary Reference Case and Scenario Results, May 2010, online: http://grist.s3.amazonaws.com/eparegs/icf-eei_scenario_results_21may2010.pdf

ICF (2010b), Clean Air, Ash and Water Regulations: Potential Impact of EPA Proposed Rules, October 2010, online: http://www.eei.org/meetings/Meeting%20Documents/2010-10-20-StrategicIssues-New%20CAA%20-%20Fine%20-%20ICF.pdf

IPCC (2005), Intergovernmental Panel on Climate Change, Carbon Dioxide Capture and Storage (IPCC Special Report), Metz, B., Davidson, O., de Coninck, H., Loos, M., and Meyer, L. (Eds.). NY: Cambridge University Press, online: http://www.ipcc.ch/pdf/special-reports/srccs/srccs_wholereport.pdf

Kaplan, S. (2010), Energy Information Administration, Potential for Displacing Coal With Generation from Existing Natural Gas Plants, presentation to the National Capitol Area Chapter, U.S. Association for Energy Economics, online: http://www.ncac-usaee.org/pdfs/2010_11Kaplan.pdf

Loulou R, Goldstein G, Noble K (2004) Documentation for the MARKAL family of models. Energy Technology Systems Analysis Programme: Paris, France http://www.etsap.org/tools.htm Accessed 29 May 2012

MJBA (2010), Ensuring a Clean, Modern Electric Generating Fleet while Maintaining Electric System Reliability,

MIT (2007), The Future of Coal: Options for a Carbon-Constrained World. Cambridge, MA: Massachusetts Institute of Technology

MIT (2010), The Future of Natural Gas, MIT Study on the Future of Natural Gas, online: http://web.mit.edu/mitei/research/studies/documents/natural-gas-2011/NaturalGas_Report.pdf

NERA (2011), Proposed CATR+MACT, online: http://www.americaspower.org/NERA_CATR_MACT_29.pdf

NERC (2010), Special Reliability Scenario Assessment: Resource Adequacy Impacts of Potential U.S. Environmental Regulation, NERC, online: http://www.nerc.com/files/EPA_Scenario_Final.pdf

NETL (2012), NETL's Carbon Storage Program, http://www.netl.doe.gov/technologies/carbon_seq/#top,

Progress in Energy and Combustion Science, Volume 35, Issue 6, December 2009, Pages 457-480

Rafaj P, Kypreos S, Barreto L (2005) Chapter 9. Flexible carbon mitigation policies: Analysis with a global multi-regional MARKAL model. In: Haurie A, Viguier L (eds) The Coupling of Climate and Economic Dynamics, Springer Verlag: New York

RFF (2009), Dallas Burtraw and Sarah Jo Szambelan, U.S. Emissions Trading Markets for SO2 and NOx, October 2009, RFF DP 09-40, online: www.rff.org/documents/RFF-DP-09-40.pdf

RFF (2011), Anthony Paul, Karen Palmer, and Matt Woerman, Clean Energy Standards for Electricity Policy: Design Implications for Emissions, Supply, Prices, and Regions, Resources for the Future, July 2011, online: http://www.rff.org/RFF/Documents/RFF-DP-11-35.pdf

Science (2002), Martin I. Hoffert, , Ken Caldeira, Gregory Benford, David R. Criswell, Christopher Green, Howard Herzog, Atul K. Jain, Haroon S. Kheshgi, Klaus S. Lackner, John S. Lewis, H. Douglas Lightfoot, Wallace Manheimer, John C. Mankins, Michael E. Mauel, L. John Perkins, Michael E. Schlesinger, Tyler Volk, and Tom M. L. Wigley, Advanced Technology Paths to Global Climate Stability: Energy for a Greenhouse Planet, Science, 298(5595): 981-987.

Simbeck and McDonald (2001), Existing coal power plant retrofit CO2 control options analysis, in: Williams, D.J., Durie, R.A., McMullan, P., Paulson, C.A.J. and Smith, A.Y. (Eds.), (IPCC 2005) Intergovernmental Panel on Climate Change (2005). Carbon Dioxide Capture and Storage, Proceedings of the 5th International Conference on Greenhouse Gas Control Technologies, Collingwood, Australia: CSIRO Publishing, pp. 103-108.

Simbeck (2001), CO2 Mitigation Economics for Existing Coal-Fired Power Plants, Presented at the U.S. Dept. of Energy National Energy Technology Laboratory (NETL), First National Conference on Carbon Sequestration, May 14-17, 2001, Washington, DC, online: http://www.netl.doe.gov/publications/proceedings/01/carbon_seq/7c2.pdf

SourceWatch (2011), Coal plant retirements, online: http://www.sourcewatch.org/index.php?title=Coal_plant_retirements#Recent_and_Upcoming_Retirements_and_Conversions

UCSUSA (2010), online: http://www.ucsusa.org/clean_energy/res/abouthow.html

Van den Broek, M. et al.(2009), Hoefnagels, R., Rubin, E., Turkenburg, W., Faaij, A., Effects of technological learning on future cost and performance of power plants with CO2 capture,

Weitzman M. (1974): Prices vs. quantities, Review of Economic Studies, 41, 477-491.

Shannon , C. E. and W. Weaver (1963), The mathematical theory of communication. University of Illinois Press, Urbana.

Shannon, C. E. (1948), A Mathematical Theory of Communication, The Bell System Technical Journal, Vol. 27, pp. 379–423, 623–656, July, October, 1948. Online: http://cm.bell-labs.com/cm/ms/what/shannonday/shannon1948.pdf

Jansen et al. (2004), Jansen, J. C., W. G. van Arkel and M. G. Boots, Designing indicators of long-term energy supply security. ECN, Petten, Netherlands, 2004. Online: ftp://ecn.nl/pub/www/library/report/2004/c04007.pdf

APERC (2007), A Quest for Energy Security in the 21st Century – Resources and Constraints. Institute of Energy Economics, Asia Pacific Energy Research Centre (APERC). Online: http://www.ieej.or.jp/aperc/2007pdf/2007_Reports/APERC_2007_A_Quest_for_Energy_Security.pdf

GEA (2012), Global Energy Assessment: Toward a Sustainable Future. IIASA, Laxenburg, Austria and Cambridge University Press, Cambridge, United Kingdom and New York, NY, USA

Stirling, A.(1994), Diversity and ignorance in electricity supply investment: Addressing the solution rather than the problem. Energy Policy, 22 (3), pp. 195–216.Online: http://www.sciencedirect.com/science/article/pii/0301421594901597

Aldy, J. (2011). Promoting Clean Energy in the American Power Sector. Hamilton Project Discussion Paper 2011-

04, May 2011. Online: http://www.brookings.edu/~/media/research/files/papers/2011/5/clean%20energy%20aldy/05_clean_energy_aldy_paper.pdf

Krupnick, A., Parry, I., Walls, M., Knowles, T., and Hayes, K. (2010). Toward a New National Energy Policy:

Assessing the Options. Resources for the Future and National Energy Policy Institute Joint Report, November

2010. Online: http://www.rff.org/Events/Pages/Toward-a-New-National-Energy-Policy-Assessing-the-Options.aspx

Palmer, K., Sweeney, R. and Allaire, M. (2010). Modelling Policies to Promote Renewable and Low-Carbon Sources of Electricity, Resources for the Future Backgrounder, June 2010. Online: http://www.rff.org/Publications/Pages/PublicationDetails.aspx?PublicationID=21281

Jansen, J. C., W. G. van Arkel and M. G. Boots, 2004: Designing indicators of long-term energy supply security. ECN, Petten, Netherlands. Online: ftp://ecn.nl/pub/www/library/report/2004/c04007.pdf

Scheepers, M. J. J., A. J. Seebregts, J. J. de Jong and J. M. Maters, 2007: EU Standards for Energy Security of Supply - Updates on the Crisis Capability Index and the Supply/Demand Index Quantification for EU-27 Energy Research Centre of the Netherlands (ECN), Clingendael International Energy Programme (CIEP), Petten. Online:

http://www.ecn.nl/docs/library/report/2007/e07004.pdf

Kruyt , et al. (2009), Kruyt, B., D. P. van Vuuren, H. J. M. de Vries and H. Groenenberg, Indicators for energy security, Energy Policy, 37(6): 2166-2181

Sovacool, B. and M. A. Brown, 2010: Competing dimensions of energy security: An

20 international perspective. Annual Review of Environment and Resources, 35(1 ): 77-108

Sovacool et al (2011), Benjamin K. Sovacool, Ishani Mukherjee, Ira Martina Drupady, Anthony L. D’Agostino, Evaluating energysecurity performance from 1990 to 2010 for eighteen countries, Energy, Volume 36, Issue 10, October 2011, Pages 5846–5853

The Clean Energy Standard Act of 2012, The US Senate Committee on Energy and Natural Resources, chaired by Senator Bingaman, 2012, online: http://www.energy.senate.gov/public/index.cfm/files/serve?File_id=8b0037eb-b801-4fc6-9918-f2c0d5da27a0

Loulou, R., G. Goldstein, et al. (2004), Documentation for the MARKAL Family of Models, IEA Energy

Technology Systems Analysis Programme. Paris.

EPA (2006), MARKAL scenario analysis of technology options for the electric sector: The impact on air

quality, U.S. Environmental Protection Agency, Washington, DC, EPA/600/R-06/114, available at:

http://www.epa.gov/nrmrl/pubs/600r06114/600r06114.pdf (last access: March 2011), 2006.

Webster, M., A.P. Sokolov, J.M. Reilly, C.E. Forest, S. Paltsev, A. Schlosser, C. Wang, D. Kicklighter, M. Sarofim,

J. Melillo, R.G. Prinn and H.D. Jacoby. 2009. Analysis of Climate Policy Targets under Uncertainty.

MIT Joint Program Report #180, MIT Joint Program on the Science and Policy of Global Climate Change,

Cambridge, MA.